28% Ultimate Bill of Materials (Darren Bowman design)

GTW 12/11/2022

All material is balsa unless otherwise specified.

National Balsa carried most for ~$175.00)

Elevator

¼ x 3/16 Stringer, 48”  7ea

3/16 center solid slab 3” x 24”

½ x 3/16 stab Leading/Trailing Edges, 48”  2ea

1/16” sheeting  4” x 36”   4ea

½ x 5/16 elevator leading/trailing edge  48”  2ea

¼ x 5/16 stringers  36”  7ea

Fuse

½ x 3/16 turtle deck stringers   36”  4ea

½ x 1” ply  12” long  1ea

¼ x 3/8 deck stringers  4 x 36”  6ea

1/8 sheet front deck  3” x 36”  2ea

¼ ply tailwheel mount   1” x 4”  1ea

1/8” balsa cabin floor, 8” sheet x 24”    (or butt glued 4” sheets)     4 sheets 1/8” x 4” x 24”

3/8 x 1” wing mount, upper  24”  1ea

1/8 x ¼ strip    36”    12ea

1/16 sheeting turtle deck rear  3” x 36”  3ea

¼ x ½ x 8” plywood firewall support, spruce  1ea

3/8” triangle stock  24”  4ea

Rudder

¼ x 3/16 stringers  6’

1/16 sheeting   3” x 36’  2ea

¼ x 5/16 stringers  36”  3ea

½ x 5/16 leading/trailing edges  48”  2ea

Wings

1/16 sheeting 4” x 36”  9ea

1/16 x 3/8 rib cap strips  24”   20ea

1/16 x 1” trailing edge   36”   10ea

½” leading wing edge  36”   8ea

7/16 x ¼ main fir/spruce spars  36”  8ea    (cut from Home Depot hemlock 7/16 x 1.5” 8 ft stick)

¾ x 2” aileron stock  36”  4ea   (make using 0.5 x 2” aileron stock, with ¼ balsa sheet backer)

½ x ¾ solid trailing edge  36”  4ea

¼ x ¼ sq wing spars, balsa   36”  20ea

Misc. Materials

DLE 55 engine

Propeller 22x8” wood

700ml gas tank

4” CF spinner

Monokote covering material (Hobby King 5 meter rolls)

Epoxy – Harbor Freight  10ea 2-tube packages

Super glue gel – Harbor Freight bulk 10 tube packages (20 each)

Super glue – Harbor Freight bulk 10 tube packages (6 each)

Denatured alcohol, for thinning epoxy to fuel proof firewall

3ea  1/16 x ½ x 36” aluminum straps, Home Depot

Dubro large scale control horns, 8ea

Aileron servo extension wires, 18” x 6ea

Radio receiver

Power switch for receiver

3000 mah LiFE battery, 6.6 volt

Servos

2 aileron 120 in-oz

2 elevator 60 in-oz

1 rudder 160 in-oz

1 throttle 60 in-oz

Pull-pull wires/kit for rudder

Metal servo pushrods with cleaves connectors  5ea

Gas engine electronic shutoff switch  1ea

Dubro pin hinges  30ea

Tygon gas line 1/8”  3 ft

Fuel dot, large  1ea

Tail wheel assembly, CF 50cc size, with 1.5” wheel

Landing gear, 50cc size CF 190mm top x 550 wide x 200mm tall

3.5” wheels, 5/32” axel spindles  2ea

Building tips

Fuselage sides and bottom are supplied (plywood), but must make fuse top from 1/8” balsa sheet, using plans as a template.  Top is similar to the bottom plywood, but longer.

Tape fuse rear together.  Locate/place first 3 bulkheads, and top and bottom fuse pieces.  Clamp. tape and rubber band together; position until square and straight over plans.   Tack glue with super glue gel.  Then go back and reinforce with 3/8” triangle stock or ¼” sq stock.

Wing 7/16” spars are not available.  Cut from Home Depot hemlock 8 ft strip of 7/16 x 1.5” stock.

Use excess 7/16” x 1.5” hemlock stock (from wing spars) to make landing gear support, and wing mounting blocks.

¾” Wing ailerons are not available.  Order ½ x 2” aileron stock, then glue ¼” x 3” balsa sheet to make a sandwich that is 0.75” thick as required

Add interior gussets to the horizontal, vertical stabs and rudder to reinforce.

Mods

- Removable top between cockpit and firewall